## Project Report: Ultrasonic Distance Measurement Tool

1. Introduction

This project aimed to develop a distance measurement tool using an HC-SR04 ultrasonic sensor and an Arduino Uno microcontroller. The tool utilizes the principles of sound waves and time-of-flight measurement to determine the distance between the sensor and an object.

2. Background: Ultrasonic Waves and Time-of-Flight

Ultrasonic waves are sound waves with frequencies higher than the human audible range (above 20 kHz). They share similar properties with audible sound waves, including reflection and the ability to travel through various mediums like air.

This project leverages the time-of-flight principle. By measuring the time it takes for an emitted sound wave to travel to an object and reflect back to the sensor, we can calculate the distance to the object. The speed of sound in air (v) is a known value (approximately 340 meters per second at room temperature).

Here's the underlying formula:

Distance (d) = (Speed of sound (v) x Time of flight (t)) / 2

The factor of 2 accounts for the round trip of the sound wave (traveling to the object and back).

3. Materials

- Arduino Uno (or compatible board)

- HC-SR05 Ultrasonic Sensor Module

- Jumper Wires

- Breadboard (optional)

4. Methodology

- An electrical circuit was constructed to connect the HC-SR05 sensor to the Arduino Uno. The Trig pin of the sensor was connected to a digital output pin of the Arduino, and the Echo pin was connected to a digital input pin.

- Arduino code was written to control the HC-SR05 sensor and calculate the distance based on the time it takes for the sound wave to travel to the object and back.

- The code triggers a short ultrasonic pulse from the sensor using the Trig pin, measures the duration of the echo pulse received back on the Echo pin, and converts that duration into a distance measurement in “cm” using the time-of-flight formula and the speed of sound.

- The measured distance was displayed on the Arduino's serial monitor for initial testing purposes.

Project Name: Ultrasonic Distance Measurement Tool

Date: [Date]

| Item Description | Quantity | Unit Cost | Total Cost |
| --- | --- | --- | --- |
| Arduino Uno R3 | 1 | [Cost] | [Total Cost] |
| HC-SR04 Ultrasonic Sensor | 1 | [Cost] | [Total Cost] |
| I2C LCD Module | 1 | [Cost] | [Total Cost] |
| Custom PCB | 1 | [Cost] | [Total Cost] |
| 9V Battery | 1 | [Cost] | [Total Cost] |
| Jumper Wires | [Quantity] | [Cost] | [Total Cost] |
| (Additional components) | [Quantity] | [Cost] | [Total Cost] |
|  |  | Total Project Cost | [Total Cost] |

Description:

This Bill of Materials (BOM) details the necessary components for the implementation of Ultrasonic Distance Measurement Tool. The Arduino Uno R3, a versatile microcontroller board, serves as the core component for program execution and control logic. The HC-SR04 Ultrasonic Sensor facilitates accurate distance measurement through ultrasonic echo detection. An I2C LCD Module is utilized for real-time display of measured distances, enhancing user interaction and feedback.

To ensure seamless integration and compactness, a Custom PCB is designed to host the circuitry, providing a structured layout and reducing wiring complexity. Power is supplied by a 9V battery, offering portability and autonomy to the system. Jumper wires and any additional components required for interfacing and functionality are accounted for in the BOM, ensuring comprehensive coverage of materials essential for the project's realization.

5. Results

The project successfully built a functional distance measurement tool. The Arduino code effectively triggered the ultrasonic sensor, measured the echo pulse duration, and calculated the distance based on the time-of-flight principle. The results were displayed on the serial monitor, providing real-time distance measurements.

6. Discussion

This project demonstrates the application of physics principles in a practical tool. The HC-SR05 sensor offers a simple and cost-effective solution for short-range distance detection using sound waves.

7. Limitations

- The accuracy of the distance measurement can be affected by factors like temperature and humidity, which cause slight variations in the speed of sound.

- The maximum measurable distance of the HC-SR05 sensor is typically around 4 meters.

- The current setup displays data on the serial monitor, which might not be ideal for all applications.

8. Future Improvements

- The project can be enhanced by incorporating an LCD screen to display the measured distance more conveniently.

- The code can be further refined to improve accuracy by incorporating temperature and humidity sensors to compensate for environmental variations in the speed of sound.

- The tool's functionality can be expanded by integrating it with other components like relays or servos for automated tasks based on distance measurements.

9. Conclusion

This project successfully developed a basic ultrasonic distance measurement tool. The knowledge gained from this project can serve as a foundation for more advanced applications using ultrasonic sensors and Arduino microcontrollers, combining principles of physics and electronics.

10. Appendix

- Include a circuit diagram (text description provided earlier).

- Include the Arduino code used in the project.

Note: This is a sample report and can be modified based on further specific research and project implementation and observations.

Code -:

#define trigPin 10

#define echoPin 9

void setup() {

Serial.begin(9600); // Initialize serial communication

pinMode(trigPin, OUTPUT); // Set trig pin as output

pinMode(echoPin, INPUT); // Set echo pin as input

}

void loop() {

long duration, distance;

// Trigger the ultrasonic sensor

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Measure the echo pulse duration

duration = pulseIn(echoPin, HIGH);

// Calculate distance in centimeters

distance = duration * 0.034 / 2;

// Print the distance to the serial monitor

Serial.print("Distance: ");

Serial.println(distance);

delay(1000); // Wait 1 second before next measurement

}